# Task 1: Network Design & IP Addressing Scheme

IT 8505 Defense-in-Depth | GulfTech Solutions | LO1, LO2 | 10 marks

## 1.1 Design decisions

Addressing model. Each site is assigned one summarisable private block from RFC 1918 space (Rekhter et al., 1996) as an administrative reservation: HQ 10.10.0.0/16, Dubai 10.20.0.0/16, Riyadh 10.30.0.0/16, Doha 10.40.0.0/16. Inside each block, VLSM/CIDR (Fuller & Li, 2006) allocates right-sized production subnets per department from the headcount and growth allowance; only these small subnets are actually consumed (Doha, for example, uses 160 of its 65,536 addresses). The larger reservation is deliberate: it preserves long-term expansion capacity and lets each site be routed as one summary (the ASA needs a single /16 route per branch). Infrastructure uses 10.255.0.0/16 (loopbacks, VPN tunnels, firewall failover link). The WAN and Internet are simulated with the three RFC 5737 documentation blocks only (Arkko et al., 2010): 198.51.100.0/24 (ISP1 /30 links), 192.0.2.0/24 (HQ perimeter 192.0.2.0/28 and ISP2 /30 links in 192.0.2.16/28) and 203.0.113.0/24 (Internet LAN). Ranges 10.10.4.0/22 (HQ) and 10.x.3.0/24 (branches) are reserved for new VLANs.

Departmental structure. The brief fixes site headcounts only (HQ 200, Dubai 150, Riyadh 120, Doha 80), so the department split below is an assumption stated for sizing; every VLAN is sized for at least 45% growth on one device per employee. HQ has HR, Marketing, Sales and R&D; branches have HR, Marketing and Sales plus a management VLAN, which keeps the same VLAN IDs (10/20/30/99) at every site for easier operation.

Firewall / DMZ placement. The Cisco ASA active/standby pair defines three zones: outside (VLAN 900, 192.0.2.0/28, shared with HQ-R1), inside (VLAN 910 transit to HQ-CORE-SW1) and DMZ (VLAN 100, its own ASA interface). The VPN router HQ-R1 sits outside the firewall, so decrypted branch traffic is still inspected by the ASA before reaching HQ. R&D (VLAN 50) stays logically isolated inside the internal zone: HQ-CORE-SW1 ACLs and the ASA both restrict it to TCP/443 towards one approved portal (NIST SP 800-41 Rev. 1 guidance on DMZ placement: Scarfone & Hoffman, 2009).

## 1.2 IP addressing and VLAN table

| VLAN / segment | Subnet | Mask | Gateway / addresses | Users* | Host capacity** | Growth headroom |
| --- | --- | --- | --- | --- | --- | --- |
| HQ VLAN 10 HR | 10.10.2.0/26 | 255.255.255.192 | .1 | 30 | 61 | +103% |
| HQ VLAN 20 Marketing | 10.10.2.64/26 | 255.255.255.192 | .65 | 40 | 61 | +52% |
| HQ VLAN 30 Sales | 10.10.1.0/25 | 255.255.255.128 | .1 | 70 | 125 | +78% |
| HQ VLAN 50 R&D (isolated) | 10.10.1.128/25 | 255.255.255.128 | .129 | 60 | 125 | +108% |
| HQ VLAN 60 Servers | 10.10.2.128/27 | 255.255.255.224 | .129 | - | 29 | - |
| HQ VLAN 99 Management | 10.10.2.160/27 | 255.255.255.224 | .161 | - | 29 | - |
| HQ VLAN 100 DMZ | 10.10.3.0/27 | 255.255.255.224 | ASA .1 (stby .2) | - | 29 | - |
| HQ VLAN 910 FW-Transit | 10.10.255.0/29 | 255.255.255.248 | Core .1 ¦ ASA .2/.3 | - | 5 | - |
| HQ VLAN 900 Outside | 192.0.2.0/28 | 255.255.255.240 | ASA .1/.2 ¦ HQ-R1 .3 | - | 13 | - |
| DXB VLAN 10 HR | 10.20.1.192/26 | 255.255.255.192 | .193 | 40 | 61 | +52% |
| DXB VLAN 20 Marketing | 10.20.1.128/26 | 255.255.255.192 | .129 | 40 | 61 | +52% |
| DXB VLAN 30 Sales | 10.20.1.0/25 | 255.255.255.128 | .1 | 70 | 125 | +78% |
| DXB VLAN 99 Management | 10.20.2.0/27 | 255.255.255.224 | .1 | - | 29 | - |
| RUH VLAN 10 HR | 10.30.1.192/26 | 255.255.255.192 | .193 | 30 | 61 | +103% |
| RUH VLAN 20 Marketing | 10.30.1.128/26 | 255.255.255.192 | .129 | 30 | 61 | +103% |
| RUH VLAN 30 Sales | 10.30.1.0/25 | 255.255.255.128 | .1 | 60 | 125 | +108% |
| RUH VLAN 99 Management | 10.30.2.0/27 | 255.255.255.224 | .1 | - | 29 | - |
| DOH VLAN 10 HR | 10.40.1.96/27 | 255.255.255.224 | .97 | 20 | 29 | +45% |
| DOH VLAN 20 Marketing | 10.40.1.64/27 | 255.255.255.224 | .65 | 20 | 29 | +45% |
| DOH VLAN 30 Sales | 10.40.1.0/26 | 255.255.255.192 | .1 | 40 | 61 | +52% |
| DOH VLAN 99 Management | 10.40.2.0/27 | 255.255.255.224 | .1 | - | 29 | - |
| FW failover link | 10.255.254.0/30 | 255.255.255.252 | FW1 .1 / FW2 .2 | - | 2 | - |
| Router loopbacks | 10.255.0.1 - .4 /32 | 255.255.255.255 | HQ .1, DXB .2, RUH .3, DOH .4 | - | - | - |
| VPN via ISP1 (per branch /30) | 10.255.1.0/30, .4/30, .8/30 | 255.255.255.252 | HQ .1/.5/.9 ¦ branch .2/.6/.10 | - | 2 | - |
| VPN via ISP2 (per branch /30) | 10.255.2.0/30, .4/30, .8/30 | 255.255.255.252 | HQ .1/.5/.9 ¦ branch .2/.6/.10 | - | 2 | - |
| ISP1 WAN links | 198.51.100.0/30 HQ, .4/30 DXB, .8/30 RUH, .12/30 DOH | 255.255.255.252 | ISP = 1st host ¦ customer = 2nd host | - | 2 | - |
| ISP2 WAN links | 192.0.2.16/30 HQ, .20/30 DXB, .24/30 RUH, .28/30 DOH | 255.255.255.252 | ISP = 1st host ¦ customer = 2nd host | - | 2 | - |
| Internet LAN (NET-INET) | 203.0.113.0/24 | 255.255.255.0 | ISP1 .1, ISP2 .2; hosts .18/.19 | - | 251 | - |

*Assumed department headcount (HQ 30/40/70/60 = 200; Dubai 40/40/70 = 150; Riyadh 30/30/60 = 120; Doha 20/20/40 = 80). **Usable addresses minus the gateway. Headroom = capacity / users - 1. Servers, management and DMZ hosts use static addresses; user VLANs use DHCP (HQ pools on HQ-CORE-SW1, branch pools on each branch router; the gateway and the first addresses are excluded).

## 1.3 Layer 2 / Layer 3 segmentation justification (every subnet)

Each subnet is an L2 segment (VLAN = its own broadcast domain). The question per subnet is where the L3 boundary sits, because that is where routing and filtering happen.

| Subnet | Segmentation type | Justification: security / routing / scalability |
| --- | --- | --- |
| HQ VLAN 10 HR 10.10.2.0/26 | L2 VLAN; L3 gateway = HQ-CORE-SW1 SVI | Security: personal data kept in its own broadcast domain; other VLANs reach it only through the routed gateway. Routing: one-hop hardware inter-VLAN routing. Scalability: 61 hosts for 30 users; new access ports join over the existing trunk. |
| HQ VLAN 20 Marketing 10.10.2.64/26 HQ VLAN 30 Sales 10.10.1.0/25 | L2 VLAN; L3 gateway = HQ-CORE-SW1 SVI | Same model as HR. Security: broadcasts and ARP/DHCP attacks stay inside the department. Routing: SVIs on the core. Scalability: /26 (61 hosts, 40 users) and /25 (125 hosts, 70 users). |
| HQ VLAN 50 R&D 10.10.1.128/25 | L2 VLAN; L3 enforced twice: SVI ACLs (RD-IN/RD-OUT) + ASA ACLs | Security: strictest isolation: only TCP/443 to one approved portal leaves; nothing enters except from the MGMT VLAN. Inside HQ it may also reach the servers VLAN on TCP/22 and TCP/443 only. Routing: still routed by the core, so policy sits at a choke point. Scalability: /25 (125 hosts, 60 users). |
| HQ VLAN 60 Servers 10.10.2.128/27 | L2 VLAN; L3 gateway = core SVI | Security: AAA (TACACS+) and IDS-management hosts are kept off user segments; branches reach it only through ASA rules (TACACS+ TCP/49, HTTPS, ICMP). Routing: static hosts, one gateway. Scalability: 29 hosts; room for syslog/DNS later. |
| HQ VLAN 99 Management 10.10.2.160/27 | L2 VLAN; L3 gateway = core SVI | Security: management plane separated from user traffic; VTY-ACL allows SSH only from the four management subnets. Routing: the ASA permits SSH to it only from the branch management subnets (plus ICMP for testing). Scalability: 29 hosts. |
| HQ VLAN 100 DMZ 10.10.3.0/27 | L3 segmentation by the firewall (own ASA interface, security level 50) | Security: public servers cannot initiate traffic to inside or branches (DMZ_IN deny). Routing: never routed by the core; gateway is the ASA (10.10.3.1). Scalability: 29 servers; a second DMZ = another ASA interface. |
| HQ VLAN 910 FW-Transit 10.10.255.0/29 | L2 transit and the inside L3 boundary | Security: only core and ASA share this link. Routing: core default route to ASA .2; ASA routes HQ subnets to core .1. Scalability: /29 holds core .1 + ASA .2/.3 (3 of 6 used). |
| HQ VLAN 900 Outside 192.0.2.0/28 | L2 perimeter segment and the outside L3 boundary | Security: contains only HQ-R1 and the ASA outside interfaces (plus NAT addresses .10/.11 of published servers); also the IDS capture point. Routing: HQ-R1 .3 to ASA .1. Scalability: 14 usable, 5 used. |
| FW failover link 10.255.254.0/30 | Dedicated L2 direct link, no gateway | Security: failover state and heartbeats never cross a production VLAN. Routing: none needed. Scalability: /30 = exactly 2 hosts. |
| Branch HR / Marketing / Sales DXB 10.20.1.192/26, .128/26, .0/25 RUH 10.30.1.192/26, .128/26, .0/25 DOH 10.40.1.96/27, .64/27, .0/26 | L2 VLANs on the branch switch; L3 gateways = branch router sub-interfaces (router-on-a-stick) | Security: one broadcast domain per department; inter-department and inter-site traffic crosses a routed hop where ACLs can be applied. Routing: the router already terminates the VPN, so no extra L3 device is needed. Scalability: a new department = a new VLAN and sub-interface. |
| Branch Management VLAN 99 10.20.2.0/27, 10.30.2.0/27, 10.40.2.0/27 | L2 VLAN; L3 gateway = branch router .1 | Security: admin hosts isolated; VTY-ACL on every device; ASA allows these subnets to SSH to HQ management. Routing: reachable via the VPN. Scalability: 29 hosts each. |
| VPN tunnels 10.255.1.0/24, 10.255.2.0/24 (/30 each) | L3 routed point-to-point (IPsec VTI) | Security: traffic is encrypted between sites. Routing: EIGRP adjacency per tunnel; ISP1 tunnels preferred, ISP2 backup. Scalability: one /30 per tunnel, no wasted addresses. |
| Loopbacks 10.255.0.1 - .4 /32 | L3 identity (no L2) | Security/Routing: stable router-ID, TACACS+ source and SSH management address that survives any single link failure. Scalability: one /32 per router. |
| WAN links and Internet LAN ISP1: 198.51.100.0/24 (4 x /30) ISP2: 192.0.2.16/28 (4 x /30) Internet LAN: 203.0.113.0/24 | L3 provider links (/30) and a shared Internet LAN | Security: the two ISPs use separate blocks (198.51.100.0/24 and 192.0.2.16/28), so every tunnel follows its own provider. Routing: static underlay with tracked default route at HQ. Scalability: /30 per link. |

## 1.4 Logical network design diagram

Figure 1.1 Logical topology of GulfTech Solutions (three zones at HQ, four VLAN sets, dual-ISP IPsec overlay). Device names, interfaces and addresses match the configuration file.

## 1.5 Implementation (configuration excerpts)

Excerpts are taken unchanged from the submitted configuration file (Sections 3, 4 and 5); lines not relevant to addressing are omitted.

### HQ-CORE-SW1 (VLANs, SVI gateways, DHCP)

vlan 10 name HR vlan 20 name MARKETING vlan 30 name SALES vlan 50 name RND-ISOLATED vlan 60 name SERVERS vlan 99 name MANAGEMENT vlan 910 name FW-INSIDE-TRANSIT ! interface Vlan10 ip address 10.10.2.1 255.255.255.192 interface Vlan20 ip address 10.10.2.65 255.255.255.192 interface Vlan30 ip address 10.10.1.1 255.255.255.128 interface Vlan50 ip address 10.10.1.129 255.255.255.128 interface Vlan60 ip address 10.10.2.129 255.255.255.224 interface Vlan99 ip address 10.10.2.161 255.255.255.224 interface Vlan910 ip address 10.10.255.1 255.255.255.248 ip route 0.0.0.0 0.0.0.0 10.10.255.2 ! ip dhcp excluded-address 10.10.2.1 10.10.2.18 ip dhcp excluded-address 10.10.2.65 ip dhcp excluded-address 10.10.1.1 10.10.1.9 ip dhcp excluded-address 10.10.1.129 ip dhcp pool HQ-HR network 10.10.2.0 255.255.255.192 default-router 10.10.2.1

### HQ-ACC-SW1 (trunk and an access port)

interface GigabitEthernet0/0 description Trunk to HQ-CORE-SW1-Gi0/2 switchport trunk encapsulation dot1q switchport trunk native vlan 999 switchport trunk allowed vlan 10,20,30,50,99 switchport mode trunk switchport nonegotiate ip dhcp snooping trust ! interface GigabitEthernet0/1 description HR-PC switchport mode access switchport access vlan 10 switchport nonegotiate switchport port-security

### DXB-R1 (router-on-a-stick gateways and DHCP; RUH-R1 and DOH-R1 follow the same pattern)

interface GigabitEthernet0/0.10 description HR encapsulation dot1Q 10 ip address 10.20.1.193 255.255.255.192 interface GigabitEthernet0/0.20 description MARKETING encapsulation dot1Q 20 ip address 10.20.1.129 255.255.255.192 interface GigabitEthernet0/0.30 description SALES encapsulation dot1Q 30 ip address 10.20.1.1 255.255.255.128 interface GigabitEthernet0/0.99 description MANAGEMENT encapsulation dot1Q 99 ip address 10.20.2.1 255.255.255.224 interface GigabitEthernet0/0.999 description NATIVE (unused, blackhole) encapsulation dot1Q 999 native ip dhcp excluded-address 10.20.1.193 10.20.1.193 ip dhcp excluded-address 10.20.1.129 10.20.1.129 ip dhcp excluded-address 10.20.1.1 10.20.1.9 ip dhcp pool DXB-HR network 10.20.1.192 255.255.255.192 default-router 10.20.1.193

### ASA-FW1 (three security zones; standby addresses belong to ASA-FW2)

interface GigabitEthernet0/0 description OUTSIDE (external zone) to HQ-PSW VLAN900 nameif outside security-level 0 ip address 192.0.2.1 255.255.255.240 standby 192.0.2.2 no shutdown interface GigabitEthernet0/1 description INSIDE (internal zone) to HQ-CORE-SW1 VLAN910 nameif inside security-level 100 ip address 10.10.255.2 255.255.255.248 standby 10.10.255.3 no shutdown interface GigabitEthernet0/2 description DMZ zone to HQ-PSW VLAN100 nameif dmz security-level 50 ip address 10.10.3.1 255.255.255.224 standby 10.10.3.2 no shutdown interface GigabitEthernet0/3 description FAILOVER LAN + STATEFUL link (direct cable FW1<->FW2) no shutdown

## 1.6 Verification evidence

Run each command after deploying the configuration and insert the capture directly under its row; the "must show" column is what the screenshot has to prove.

| ID | Command(s) and device | The capture must show |
| --- | --- | --- |
| E1.1 | HQ-CORE-SW1, HQ-ACC-SW1, branch switches: show vlan brief | VLANs 10/20/30/50/60/99/910 (HQ) and 10/20/30/99 (branches) with the names and ports in the table; unused ports in VLAN 998. |
| E1.2 | HQ-CORE-SW1: show ip interface brief ¦ include Vlan | Vlan10 .2.1, Vlan20 .2.65, Vlan30 .1.1, Vlan50 .1.129, Vlan60 .2.129, Vlan99 .2.161, Vlan910 10.10.255.1: all up/up. |
| E1.3 | All switches: show interfaces trunk | Allowed VLANs match the plan; native VLAN 999; no DTP (nonegotiate). |
| E1.4 | Each branch router: show ip interface brief and show ip route connected | Gi0/0.10/.20/.30/.99 with the gateway addresses, Gi0/1 and Gi0/2 WAN addresses, Loopback0, Tunnel1/Tunnel2. |
| E1.5 | ASA-FW1: show nameif, show interface ip brief, show failover | outside 192.0.2.1, inside 10.10.255.2, dmz 10.10.3.1; standby .2, .3, .2 on ASA-FW2; three zones present. |
| E1.6 | HQ-CORE-SW1: show ip dhcp binding; branch routers: same | Leases inside each VLAN subnet, none overlapping excluded addresses (expected first leases: HR .19, MKT .66, Sales .10, R&D .130 at HQ; if boot order changes them, update the diagram to the real leases). |
| E1.7 | HR-PC: ping 10.10.2.66, trace 10.10.2.66; DXB-HR-PC: ping 10.20.1.130 | Inter-VLAN routing works between HR and Marketing at HQ and at Dubai; the trace shows the VLAN gateway as the first hop. |
| E1.8 | DMZ-SRV: ping 10.10.3.1; TACACS-SRV: ping 10.10.2.129 | DMZ gateway is the ASA; server VLAN gateway is the core. (Cross-site tests belong to Task 3; filtering tests to Tasks 2 and 5.) |

[ Insert screenshots E1.1-E1.3: VLAN, SVI and trunk output ]

[ Insert screenshots E1.4-E1.5: branch router and ASA addressing output ]

[ Insert screenshots E1.6-E1.8: DHCP leases and ping/trace results ]

## 1.7 Analysis

The scheme gives every department at every site its own VLAN and subnet, keeps at least 45% spare capacity per user VLAN, and leaves each site's /16 largely free for new VLANs. Placing the DMZ on a dedicated firewall interface, R&D behind two enforcement points and the VPN router outside the ASA means each trust boundary is a routed, filterable hop. Residual limitation: branches rely on router-based gateways, so inter-VLAN filtering at branches is a later enhancement (recommended in Task 8). Record any addressing issue found during deployment and its fix here, as Task 7 requires.

### References (Task 1)

Arkko, J., Cotton, M., & Vegoda, L. (2010). IPv4 address blocks reserved for documentation (RFC 5737). Internet Engineering Task Force. https://www.rfc-editor.org/rfc/rfc5737

Fuller, V., & Li, T. (2006). Classless inter-domain routing (CIDR): The Internet address assignment and aggregation plan (RFC 4632). Internet Engineering Task Force. https://www.rfc-editor.org/rfc/rfc4632

Rekhter, Y., Moskowitz, B., Karrenberg, D., de Groot, G. J., & Lear, E. (1996). Address allocation for private internets (RFC 1918). Internet Engineering Task Force. https://www.rfc-editor.org/rfc/rfc1918

Scarfone, K., & Hoffman, P. (2009). Guidelines on firewalls and firewall policy (NIST Special Publication 800-41 Rev. 1). National Institute of Standards and Technology. https://doi.org/10.6028/NIST.SP.800-41r1